# SkiSafe — Engineering Notebook

Hardware Sensor & Physical Integration Log

Week of March 15 – 21, 2026  |  Engineer: Diego Anderson

Project Overview

| Project | SkiSafe — Adaptive Helmet & Rear-Detection System for Cochlear Implant Users |
| --- | --- |

| Scope of this log | Hardware sensor integration, OLED display, haptic feedback planning, and helmet mounting strategy. Excludes TinyML / Edge Impulse work. |
| --- | --- |

| Hardware stack | Adafruit Feather M4 Express (ATSAMD51) + FeatherWing OLED 128x64 + VL53L1X Time of Flight Sensor + LiPo battery |
| --- | --- |

| IDE | Arduino IDE |
| --- | --- |

Entry 1 — Project Setup & Hardware Overview

| Mar 15–16 | Chat initialized — hardware scope defined |
| --- | --- |

| Status | Feather M4 Express and FeatherWing OLED in hand and tested. VL53L1X ToF sensor ordered, arriving later in week. |
| --- | --- |

| Scope confirmed | - This chat owns: sensor code, OLED display code, haptic feedback, wiring, mounting, and physical helmet integration. - Separate chat handles: TinyML, Edge Impulse, FOMO model, Arducam, Arduino Nano 33 BLE Sense. - Split rationale: two independent systems that will eventually communicate but can be developed in parallel. |
| --- | --- |

| Hardware confirmed | - Feather M4 Express (ATSAMD51 Cortex M4) — main processor - FeatherWing OLED 128x64 (STEMMA QT) — soldered directly on top of Feather M4 - VL53L1X Time of Flight Sensor (30–4000mm, STEMMA QT) — rear detection - LiPo battery — power source for the stack |
| --- | --- |

| Key note | All three devices communicate over I2C. FeatherWing stacked on Feather M4 shares the I2C bus. VL53L1X connects via STEMMA QT (~10cm cable). No breadboard used. |
| --- | --- |

| Design note | Primary user population: cochlear implant users, hearing aid users, deaf individuals, women and young girls. Engineer (Diego) wears cochlear implants — firsthand user perspective informs design. |
| --- | --- |

Entry 2 — Advisor Meeting & Hardware Integration

| Mar 17–18 | All three components connected — ToF readings on OLED confirmed |
| --- | --- |

| Milestone | Met with faculty advisor. Successfully connected Feather M4 + FeatherWing OLED + VL53L1X ToF sensor together for the first time. |
| --- | --- |

| Accomplished | - Wired all three components together during advisor meeting. - VL53L1X distance readings displaying live on the 128x64 OLED screen. - Readings described as semi-accurate — functional proof of concept confirmed. - Physical stack is compact: FeatherWing soldered on Feather M4, ~10cm STEMMA QT cable to ToF sensor, powered by LiPo. |
| --- | --- |

| I2C note | All three devices on shared I2C bus. No address conflicts observed. Worth verifying VL53L1X vs OLED default I2C addresses in next session. |
| --- | --- |

| Next focus | Improve sensor accuracy (code), plan physical helmet mounting, and source haptic motor. |
| --- | --- |

Entry 3 — Helmet Research & Mounting Strategy

| Mar 19–20 | Helmet candidates identified — mounting approach under discussion |
| --- | --- |

| Helmets under consideration | Giro Ledge MIPS (REI, hard ABS shell, clean rear geometry) and Scott Flow Pro MIPS (Scott Sports, more aggressive ventilation channels at rear). |
| --- | --- |

| Mounting constraints identified | - ToF sensor requires clear rear line-of-sight — must mount at back of helmet. - Feather M4 + OLED stack is brain + debug display only; haptic feedback is the primary user alert. - ~10cm STEMMA QT cable limits distance between sensor and brain stack. - LiPo battery already integrated into the stack — no separate battery housing needed for v1. |
| --- | --- |

| Mounting approach options discussed | - 3D print a custom enclosure (engineer has prior 3D printing experience, school printer available). - Adapt an existing design from Thingiverse. - Velcro / adhesive mounting for rapid first prototype. - Decision deferred pending physical helmet measurements. |
| --- | --- |

| Teacher suggestion | Design or adapt a case using Thingiverse as a starting point, then 3D print a custom-fit mount. |
| --- | --- |

| Action item | Physically measure the flat rear zone of both helmets. Identify viable mounting surface dimensions before designing enclosure. |
| --- | --- |

Entry 4 — Haptic Feedback Planning

| Mar 21 | Haptic system scoped — wrist placement favored for v1 |
| --- | --- |

| Key decision | Haptic feedback is the primary alert mechanism for the target user population (deaf / hard of hearing). OLED display is a secondary / debug output. |
| --- | --- |

| Haptic motor status | - Engineer may have vibration motors at home — to be confirmed. - Teacher noted resistors and transistors will likely be needed to drive motor from Feather M4 GPIO. - Motor type and part number TBD pending inventory check. |
| --- | --- |

| Placement discussion | - Alert does not have to be helmet-mounted — wrist placement discussed as potentially more reliable. - Wrist vibration is easier to feel consistently vs. helmet vibration, especially at speed with wind. - Final placement TBD — wrist is the leading candidate for v1 prototype. |
| --- | --- |

| Next steps (immediate) | - Find vibration motors at home — note part number or description. - Physically measure rear of both helmets for mounting surface dimensions. - Research transistor-based motor driver circuit for Feather M4. - Begin enclosure design once helmet measurements are in hand. |
| --- | --- |

| Prototype philosophy | V1 goal: prove the concept works physically. Elegance comes later. A working ugly prototype beats a beautiful non-functional one. |
| --- | --- |

Open Items & Questions

| # | Item | Status |
| --- | --- | --- |
| 1 | Confirm vibration motor part number / type | Pending — check at home |
| 2 | Measure rear of Giro Ledge + Scott Flow helmets | Pending |
| 3 | Design or source 3D printable enclosure | Blocked on #2 |
| 4 | Verify I2C address map (VL53L1X vs OLED) | To do — next code session |
| 5 | Research transistor motor driver circuit for Feather M4 | To do |
| 6 | Improve ToF sensor reading accuracy | To do — next code session |
| 7 | Decide final haptic motor placement (wrist vs helmet) | In progress |

End of week — March 15–21, 2026